Ali Raza

9864

BScs-Section (A)

Coal –A

Lab No 01

- Task no 01:

Start the computer; go to BIOS menu by pressing F2 key on the keyboard. Record information

like

a. Motherboard’s model and manufacturer

b. Microprocessor’s model and manufacturer

c. Installed RAM

d. Installed hard disk drives etc.

Motherboard’s model and manufacturer

 Manufacturer: Dell Inc.

- Model: Dell OEM board

- Microprocessor’s model and manufacturer:

Intel(R) Core(TM) i7-2600 CPU @ 3.40GHz
Manufacturer: Intel

Installed RAM:
16384 MB (16 GB) DDR3 SDRAM
DIMM 1 Size = 8192 MB
DIMM 2 Size = 8192 MB
Memory Speed = 1333 MHz
Channel Mode = Dual Interleave

Installed hard disk drives etc:
SATA-0 = {none}
SATA-1 = INTEL SSDSA2BW120G3 (SSD)

- Task no 02:

Switch off the computer. Remover the power supply chord from the power socket. Open casing

of PCs available in the lab.

- Task no 03:

Discover about the power supply in the chassis. Record the type, model, power rating of the

supply and the available different voltage levels.

- Power Supply Details

1. Type

- Form Factor: ATX (standard desktop size)

- Design: Dell OEM internal PSU

- Cooling: Built-in fan with rear exhaust

2. Model

- Dell systems with BIOS version A10 and Intel i7-2600 typically use:

- Model: Dell L250E-S00 or L300E-S0 (varies by exact chassis)

- These are Lite-On or Delta Electronics manufactured units for Dell

3. Power Rating

| Model | Wattage | Typical use |
| --- | --- | --- |
| L250E-S00 | 250W | Entry-level desktops |
| L300E-S0 | 300W | Mid-range desktops |

 Efficiency: Usually 80 PLUS Bronze certified

- Connector Types: 24-pin ATX, 4-pin CPU, SATA power, Molex

4. Voltage Rails Provided

| Voltage Rail | Purpose |
| --- | --- |
| +3.3V | Motherboard logic, RAM |
| +5V | Legacy devices, USB power |
| +12V | CPU, GPU, fans, drives |
| -12V | Serial ports, legacy support |
| +5VSB | Standby power (wake-on-LAN) |

- Task no 04:

Discover about the motherboard chipset. Download its datasheet using internet. Match the

information found in the datasheet by physical inspection of the motherboard.

- Identify the Chipset

- processor: Intel Core i7-2600

- Compatible chipsets: Intel H61, H67, P67, Q67

- Dell business desktops (2011–2013) typically use: Intel Q67 Express Chipset

- Datasheet Information

Key features from the official Intel datasheet:

- Memory Support: DDR3 SDRAM up to 1333 MHz (matches your 16 GB DDR3 @ 1333 MHz)

- Storage Interfaces: SATA 3 Gb/s ports (your SSD is connected via SATA-1)

- PCI Express: Supports PCIe x16 slot for graphics and PCIe x1 slots for expansion (your BIOS shows empty PCI slots)

- Integrated Graphics: Intel HD Graphics support (matches your Video Controller = Intel HD Graphics)

- Physical Inspection Match

- Chipset marking: A small square IC near the CPU socket labeled “Intel Q67” or similar.

- RAM slots: Two DDR3 DIMM slots (you have 2 × 8 GB installed).

- SATA connectors: At least 2–4 SATA ports on the board (you saw SATA-0 empty, SATA-1 SSD).

- PCIe slot: One long slot (x16) for GPU, plus shorter slots (x1).

- I/O Ports: VGA, USB, Ethernet, Audio — all visible on the back panel.

- Task no 05:

Record type and number of following interfaces; Find their brief description from datasheet.

a. PS/2

b. USB

c. Serial

d. Parallel

e. PCI

f. PCIe

g. AGP

h. VGA

i. HDMI

j. Audio

k. Ethernet

l. SATA and PATA

| Interface | Type/Count (Dell Q67 chipset board) |  | Brief Description |  |
| --- | --- | --- | --- | --- |
| PS/2 | 2 (Keyboard + Mouse) |  | Legacy round connectors for input devices |  |
| USB | 6–8 ports (USB 2.0) |  | Universal Serial Bus for peripherals like flash drives, printers |  |
| Serial | 1 (optional, rear I/O) |  | COM port for legacy devices (modems, industrial equipment) |  |
| Parallel | 1 (optional, rear I/O) |  | Legacy printer port |  |
| PCI | 1 slot |  | Expansion slot for older cards |  |
| PCIe | 1 ×16 (graphics), 2 ×1 |  | High-speed expansion slots for GPU, NIC, etc. |  |
| AGP | None |  | Legacy graphics slot (not present in modern boards) |  |
| VGA |  | 1 port |  | Analog video output |  |
| HDMI |  | 1 port (depends on model) |  | Digital video/audio output |  |
| Audio |  | 3 jacks (Line-in, Line-out, Mic) |  | Conexant HDA CX2D641 controller |  |
| Ethernet |  | 1 port |  | RJ-45 LAN port for networking |  |
| SATA and PATA |  | SATA: 2–4 ports, PATA: None |  | Storage interfaces for HDD/SSD |  |

- Task no 06:

Discover about the micro-processor and the socket type in which it is installed on the

motherboard.

Microprocessor Model & Manufacturer:

- Intel® Core™ i7-2600 CPU @ 3.40 GHz

- Manufacturer: Intel Corporation

Key Specifications (from Intel datasheet):

- Architecture: Sandy Bridge (32nm process technology)

- Core Count: 4 cores, 8 threads (Hyper-Threading enabled)

- Base Clock Speed: 3.40 GHz (Turbo Boost up to 3.80 GHz)

- Cache: L2 = 1 MB, L3 = 8 MB

- Instruction Set: 64-bit (Intel EM64T), SSE4.2, AVX

- Integrated Graphics: Intel HD Graphics 2000

- Thermal Design Power (TDP): 95W

Socket Type:

- Installed in Socket LGA1155 (Socket H2)

- This socket supports Intel’s 2nd Generation Core processors (Sandy Bridge series).

- Physical inspection: Square socket with a lever lock mechanism, located under the CPU cooler/fan on the motherboard.

- Task no 07:

Shed light on the memory hierarchy being employed in this PC. Fill the following table.

Memory type

Size

Description

Registers

Cache

RAM

Disk storage

| Memory Type | Size | Description |
| --- | --- | --- |
| Registers | Few bytes (per core, typically 32 × 64-bit registers) | Fastest storage inside CPU, used to hold instructions and operands during execution. |
| Cache | L1: 64 KB per core (32 KB data + 32 KB instruction) L2: 256 KB per core (total ~1 MB) L3: 8 MB shared | High-speed memory located inside CPU. Stores frequently accessed data/instructions to reduce latency. |
| RAM | 16 GB DDR3 SDRAM @ 1333 MHz | Main system memory. Holds active programs and data currently in use. Volatile (data lost when power off) |
| Disk Storage | 120 GB Intel SSD (SSDSA2BW120G3) | Non-volatile storage. Stores OS, applications, and user data permanently. Much slower than RAM but faster than HDD due to SSD technology. |