Lab No 08

Lab Work

- Write the following code and explore functioning of procedure and stack by debugging it:

.686p

.MODEL flat,stdcall

ExitProcess PROTO, dwExitCode:DWORD ; to use this proto, .model must be stdcall

.data

val1	DWORD  10000h

val2	DWORD  40000h

val3	DWORD  20000h

sum	DWORD  ?

.CODE

main PROC

mov	EAX, 	40		; ESP=?			[ESP] = ?

mov	EBX, 	10

mov	ECX, 	-20

call	sumof 			; ESP=?			[ESP] = ?

mov	sum, 	EAX		; ESP=?			[ESP] = ?

invoke	ExitProcess, 0	; to terminate the program

main ENDP

sumof PROC

push	EAX		; ESP=?			[ESP] = ?

add	EAX, EBX	; ESP=?			[ESP] = ?

add	EAX, ECX

pop	EDX

ret			; ESP=?			[ESP] = ?

sumof	 ENDP

END main

Output:

# Exercise

- Use stack to swap the value of two integer variables.

Program:

.686p

.MODEL flat,stdcall

ExitProcess PROTO, dwExitCode:DWORD

.data

val1 DWORD 5

val2 DWORD 10

.code

main PROC

mov EAX, val1

mov EBX, val2

push EAX

push EBX

pop EAX

pop EBX

mov val1, EAX

mov val2, EBX

invoke ExitProcess, 0

main ENDP

END main

Output: